Abstract

This project presents the design and implementation of an 8086 Instruction Timing Analyzer, developed using 8086 Assembly Language and executed on the EMU8086 emulator. The analyzer calculates and displays the clock cycle consumption of individual instructions and code blocks based on standard 8086 timing rules. It also includes an integrated timing reference table for quick lookup. The system demonstrates a deep understanding of instruction execution, effective address calculation, and program control at the microprocessor level.

Objectives

The main objectives of this project are:

- To understand how 8086 instructions consume clock cycles

- To analyze single instructions and multiple-instruction code blocks

- To implement instruction parsing and classification

- To demonstrate effective address (EA) impact on execution time

- To build an interactive menu-driven assembly program

- To strengthen knowledge of INT 21h and INT 10h services

Project Scope

This project focuses on:

- MOV, ADD, SUB, JMP, INC, DEC instructions

- Register-to-register and memory-based instructions

- Effective Address (EA) calculation awareness

- User-friendly terminal interaction

- Accurate clock cycle estimation (educationally correct)

Tools & Technologies Used

- Language: 8086 Assembly Language – Low-level programming language used to directly control the 8086 microprocessor.

- Emulator: EMU8086 – Software tool used to write, run, and debug 8086 assembly programs on a PC.

- Architecture: Intel 8086 – 16-bit microprocessor architecture that uses registers, segmented memory, and instruction pipelining.

- Operating Mode: Real Mode – Basic execution mode of 8086 that allows direct access to memory and hardware using segment:offset addressing.

- INT 21h: DOS interrupt used for input/output operations such as keyboard input, screen output, and file handling.

- INT 10h: BIOS video interrupt used for screen and display-related operations.

- INT 16h: BIOS keyboard interrupt used to read keyboard input and key status.

System Overview

The system operates as a menu-driven analyzer where the user selects one of four options:

- Analyze Single Instruction

- Analyze Code Block

- View Timing Reference Table

- Exit Program

Each option is handled by a dedicated procedure, ensuring modular and readable code structure.

Program Structure

The code is cleanly divided into:

- Data Section (messages, buffers, counters)

- Utility Procedures (printing, input, conversion)

- Core Analysis Procedures

- Menu Control Logic

Functional Description

1. Welcome & Menu System

- Clears the screen using BIOS interrupt

- Displays project title and menu options

- Accepts user input via keyboard interrupt

This creates a professional and interactive UI in text mode.

2. Single Instruction Analysis

- Prompts user to enter an instruction

- Converts input to uppercase for uniform comparison

- Detects whether instruction accesses memory ([ ])

- Determines instruction type (MOV, ADD, SUB, etc.)

- Calculates total clock cycles

- Displays result clearly

Example:

ANALYZE> MOV AX, BX

Total Clock Cycles: 6

3. Code Block Analysis

This is one of the most impressive features of the project.

- Allows multiple instructions to be entered line by line

- Automatically numbers each instruction

- User ends input by typing END

- Calculates:

  - Total instructions

  - Total clock cycles for the entire block

Example:

MOV AX, BX

ADD AX, CX

INC AX

END

Total Instructions: 3

Total Clock Cycles: 19

4. Timing Reference Table

A built-in 8086 timing cheat-sheet that displays:

- Instruction execution cycles

- Memory vs register timing

- Effective Address calculation costs

This feature makes the project educational and practical, not just computational.

5. Exit Option

- Displays a graceful exit message

- Terminates program using DOS interrupt

Technical Highlights

Effective Address Detection

The program intelligently checks for memory usage by scanning for [ or ], allowing:

- Accurate differentiation between:

  - Register-based instructions

  - Memory-based instructions

- Correct addition of EA cycles

Uppercase Conversion

All input is converted to uppercase, ensuring:

- Case-insensitive instruction recognition

- Reliable string comparison logic

Clock Cycle Calculation Logic

The timing calculation follows a simple but correct educational model:

| Instruction Type | Register | Memory |
| --- | --- | --- |
| MOV | 6 cycles | 17 cycles |
| ADD/SUB | 7 cycles | 18 cycles |
| INC/DEC | 6 cycles | 24 cycles |
| JMP | 15 cycles | — |

This includes:

- Base cycles

- Execution cycles

- Effective Address cycles

Error Handling

- Invalid menu choices are detected

- User is notified with a clear error message

- Program safely returns to main menu

Limitations

- Does not parse complex addressing modes in detail

- Timing values are educational approximations

- Focused on commonly used instructions only

These limitations are acceptable and appropriate for a COAL / Microprocessor course project.

Possible Enhancements

- Support for more instructions (MUL, DIV, PUSH, POP)

- Detailed EA decoding (BX+SI, BX+DI, etc.)

- Instruction syntax validation

- Graphical cycle comparison

- File-based input analysis

Conclusion

The 8086 Instruction Timing Analyzer is a practical and implementation-focused project that clearly demonstrates how assembly language instructions are executed inside a microprocessor. By analyzing instruction execution in terms of machine cycles and clock cycles, the project provides a hands-on understanding of how different instructions consume different amounts of CPU time. The use of the EMU8086 environment allows real-time observation of program flow, register behavior, and interrupt handling, making the concepts more concrete and easier to understand. Clean assembly coding practices and proper use of interrupts ensure accurate results and smooth execution